MID Module on-board computer - User manual

Controls:

Control module on-board computer is the same as for standard MID, ie, two buttons on the wiper control lever at the controls. Press. Set display parameter selection is performed, reset button resets the parameter. After pressing the SET button is on display now shows the type parameter, and consequently its value.

Current Consumption:

Immediate consumption is displayed in liters per hour, if the vehicle speed above 15 km per hour in liters per 100 km.

Average consumption:

Average consumption is calculated by the amount of fuel consumed and distance traveled since last reset this parameter. There are two, independent of each measurement. Switching is accomplished by pressing the reset button. To indicate the display of the second measurement is available arrow symbol is displayed before the average in the second row . The second measurement (marked by arrow) is stored after removing the ignition key into the internal memory is saved and remains even after removing the batteries . To reset the parameter is necessary to hold the reset button for longer than the three seconds.

Fuel consumed:

Average speed:

Distance traveled:

The amount of fuel used , average speed and distance traveled is counted as well since the last resetting of the parameter . There are again two independent measurements of each other . Switching is accomplished by pressing the reset button. To indicate the display of the second measurement is available arrow , shown before the feature parameter in the second row . The second measurement (marked by arrow) is stored after removing the ignition key into the internal memory is saved and remains even after removal autobaterie.K reset parameter is necessary to hold the reset button for longer than the three seconds.

Vehicle Range:

Measurement of vehicle roll can be done two different ways according to the method of resetting this parameter. After holding the reset button for longer than three seconds a menu will appear with a choice of whether or not pumped full tank.

When selecting a full tank is calculated with the initial roll-out a defined amount of a full tank, which can be set in the service menu and the calculation is made according to fuel consumption. This method of measurement is more accurate, since it bypasses the sensor measuring the amount of fuel in the tank, the input A / D converter and any tilt.

Unless you select a full tank, the roll counts after measuring the amount of fuel in the tank. To measure the condition of each tank is inserted in the ignition key to active position.

Range calculated with an average consumption of vehicles in the last 30 km. Views of consumption over the last 30 km is displayed every minute for 5 seconds if the function displayed just roll out of the vehicle.

Stopwatch:

Controls the function by pressing the reset button, which stems run, stop and reset.

Tank Status:

Measuring the amount of fuel in the tank is performed continuously and is displayed in liters on the display. It should be noted that the measurements will always be vitiated due to following reasons: accuracy of the sensor in the tank and its wear tolerance, input circuits, A/D converter, tilt, movement of vehicles. The size of the tank can be set in the service menu.

Vehicle speed:

Engine speed:

Since the parameters can be measured for proper function onboard computer, there was reason to suppress the display and we use the indicators in monitoring the accuracy of onboard instruments.

Measuring battery (car battery):

Measurement of electrical system is necessary for correct measurement of the tank and use it to the user in proper control of battery charging. Ideally, the voltage at engine start-up ranges from about 13.7V to 14.3V. A significant difference in mean failure in the electrical system. Module on-board computer is also equipped with automatic controls charging car batteries, which is set to range from 12.5V to 15.5V. If you exceed this range, the user is informed of the warning sign on the display.

Internal temperature:

After holding the reset button we have a set menu that allows climate control. There is setting the desired temperature and the so-called Hysteresis. Hysteresis setting is important because in order to avoid frequent switching on and off the air conditioning compressor is not designed for it.

Example:

Set the temperature of 23 ° C and 2 ° C. Hysteresis When this setting is to turn on air conditioning at 25 ° C and off at 21 ° C.

Immobilizer:

Immobilizer function can be turned on / off service menu. If you use the immobilizer and the connection of an external relay is required after turning the key in the ignition in the active position to enter a PIN. Otherwise, the vehicle can start. PIN is specified using the control lever and the following way:

Example: (PIN 23)

2 x press the SET button, press button 1 x Reset

3 x press the SET button, press button 1 x Reset

Press 1 x reset button

PIN can be of one to four numbers. PINs are used and two administrators and users. Using the admin PIN can deactivate the immobilizer and enter the service menu. PIN using the user can disable the immobilizer, but you can not enter into the service menu.

Number One Administrator PIN value is No. 3

Day lighting:

Output daytime running lights are activated when starting the engine and disable the engine after shutdown.

Setting the on-board computer:

After holding the SET and RESET buttons for more than 3 seconds when we get to the menu with the following choices :

- Turn off the immobilizer

- Select the user menu
- Service menu
- Set the time
- Adjust the display brightness

Turn off the immobilizer :

In case of installation and use of the immobilizer this feature to temporarily shut down before sliding the immobilizer ignition key in order to avoid re- enter the PIN.

Selecting the user menus:

Given the large number of parameters can be displayed in the service menu (user menu) option set , which features the display will show , because not everyone has all the parameters of interest . You then select which menu is displayed :

- "A" All the functions

- "B " Only the selected function

Service menu:

Menu určené k nastavení palubního počítače. Pro vstup je nutné zadat hlavní kód PIN.

Set Time:

There you have set hours. SET button selection is performed , reset button perform parameter adjustment .

Adjust the brightness of the display:

LCD brightness can be adjusted in eight steps and one for the lights on and off (day / night). SET button selection is performed , reset button perform parameter adjustment .

Service menu :

Service menu to configure the onboard computer according to your preference with subsequent options:

- Select the language (Czech, Slovak, English, Polish, Hungarian)

- Set security codes
- Setting the immobilizer
- Adjust the tire size and number of pulses per revolution of the wheel
- 1 km to the impulse tachometer
- Select engine control unit
- Setting up user menu
- Adjust the size of the tank
- Correction of consumption
- Display data from radio
- Selection of the parameters displayed on the display
- Control function
- Other functions
- Reset BC (Load basic parameters)

In the service menu using the SET parameter selection is performed , reset button make changes or confirm the selection. One choice is always either just "Go Back" without saving the parameters , or " Return and Save ."

Choose the language:

There is a choice of five languages ​​:

- Czech
- Slovak
- English
- Polish
- Hungarian

View all supported languages ​​with diacritical marks diacritical marks these options off. This option is to install a custom display case with a different character set , which would cause an absurd display of characters on the screen.

Set security PIN:

When selecting a PIN change first choose who we want to change your PIN (admin / user). After selecting a value enter the PIN as it is written above item immobilizer. To check, enter the value again. If the values are the same, the new PIN is stored, otherwise the warning message is printed.

Immobilizer:

An immobilizer is possible in the service menu and turn off and then there is the option of switching or relay output expansion mode, the immobilizer. When selecting a switch mode output relay is energized if the correct PIN (we use that option in case if we want to block as a starter). Expanding the system will be used when blocking as the fuel pump. In this case, the relay activates (opens) when the engine is started without entering a PIN.

Set of tire sizes:

In this case we have to choose the exact size of the tires, which will provide us with very real value of the distance and the proper metering. If we ask the same value as a distance indicator on the dashboard instruments we choose to see ”according to impulse.“. In this case, the on-board computer to count from the preset value of pulses per 1km. But the setting is usually loaded error around 2%!

Impulse Odometer:

If as stated above, use the " pulse according to " Set here the number of pulses that traveled for 1km. The value of this parameter, we find holding the button. reset daily kilometers to flight instruments while turning the key in the ignition to the active position. For vehicles Astra G it is usually worth the 15385. For vehicles Frontera , it is the value of the 2700.

The choice of engine control unit:

In this menu, you select the type of control unit , which corresponds to the vehicle in which the BC module is used. In the case of connecting the module to one of the injectors , use the " injection valve" . Optocoupler connection diagram is located on the web page.

User Settings menu:

The module allows on-board computer to switch two different menus. Menu ”A“ shows all the parameters. Menu ”B“ only selected. Parameters displayed in the ”B“ can be set here so you see only those parameters that really matter to us.

Tank Size:

The size of the tank car Opel Astra G is about 51 liters. In this case we have the opportunity to edit and select the correct indicator of vehicle roll.

Correction of consumption:

Power measurement is essentially based on the time of opening the fuel injection valve engine. Theoretically, fuel linearly dependent on the injection valve opening time, provided always the same pressure fuel system. In reality, though, the pressure in the fuel system changes, even in close tolerance and the impact it has on many factors. The quality of fuel and its density, and fuel pump wear, the state of the fuel filter and fuel systems, engine load, etc.. It should also be taken into account for close / open valve injection. All inserted in the measurement of the number of variables, which in the end shows the difference of measured and actual consumption, as I have myself verified by testing on several vehicles with the same engine. The measured difference is not large, mostly reflected the difference in about one to two tenths of average consumption.

For this reason, the Service menu item added adjustment of consumption, which can be adjusted to the vehicle itself. The difference simply find before tank full tank of fuel consumed by resetting the counter on-board computer and then starting off the tank. Then again refuel a tank full of fuel and by tank indicators and on-board computer, find the difference. 

 	The amount of fuel pumped then divided by the amount of fuel on-board computer and enter the result rounded up to the adjustment of consumption items.

Radio show:

When using the original radio set here all what we wish to be displayed. Symbols of radio is displaying on the third line, RDS appear in the first two thirds of the fourth row. If you turn off display of RDS car radios, are displayed instead of two optional parameters measured . In the case of the third row we have three options and display symbols of radio, display symbols instead of three optional parameters measured here, or show only dashes to separate (greater clarity).

View the display of optional parameters:

The display can be set up and display up to eight of all measured parameters of the onboard computer . Display functions are labeled Dis-0 to Dis-7. Function Dis-0 is located at the end of the line features, Dis-1 at the end of the second line, function Dis-2, Dis -3, Dis-4 on the third row and function Dis-5, Dis -6, Dis -7 on the line fourth. Set button to select the displayed function will be set and reset button as it will set.

Control functions:

The various control functions in this menu, you can independently adjust everything according to what we want to check and control sensors that are installed on the vehicle. SET button to select the desired function and press the RESET switch is performed, or disable this function .

Other functions:

In this menu we can activate a relay output for daytime running lights , the choice on what vehicle will be used to display the MID (Astra G or Frontera ) and set the warning temperature from external temperature sensor .

Configuration of the initial inscription:

After ignition , the display will start writing for a while . This sign can be changed to suit your requirements. SET button to select the desired function and position of letters and reset button are changing the letters , save banner or go back.

Resetting the Board Computer:

The implementation of this option On - Board computer is set to the default factory settings.

Any other questions like the answer at veramon.astra@seznam.cz