Installation of the On-Board computer Astra G

(MID)

An onboard computer is different depending on whether the vehicle was originally installed MID (32-pin connector), or only TID (12-pin connector). In the case of exchange of the MID is not necessary to install any additional wiring is performed only change the display. In the case of exchange of the TID is necessary to install the necessary wires that are on a volume not to TID display .

Before installation, disconnect the car battery while the car battery is disconnected and the involvement of the vehicle must not be any person !!!

Removing the center console:

- Remove the radio and storage space (the drawer under the car radio)

- Remove ashtray and remove the cover of a car cigarette lighter

- Allow the screws under the cover of a car ashtray and lighter

- Remove the metal shaft radio (on the back is screw)

- Cut to the car stereo connectors, including the antenna connector

- Allow the screws on the sides, where they found the shaft radio

- Disconnect the control air-conditioning (2 clips at the top and bottom 2 clips)

- Pull switch to remove the directional warning lights

Now we no longer cover the entire center console holds the upper metal clip, making it stronger by pulling out and then pull out.

- Push out the plug warning light

- Disconnect connector from the display TID

Removing the panel board instruments:

- Take off two covers at the bottom of the flight instrument

- Allow four screws (2 below and 2 above)

- Push out the plastic cover on-board instruments

- Allow the screws that hold the board device (2 below)

- Disconnect connector (top left of the lever is plastic)

- Remove the panel board instruments

- Remove the connector cable assembly from the cradle

To remove the connector to the cable assembly from the holder, it is necessary to squeeze plastic holder at the bottom left downwards. Connector beam can then be easily pulled to the left out

Removing control:

- Pull the two plastic screw covers on the steering wheel (it is necessary to turn the steering wheel)

- Unscrew the screws underneath these covers

- Towards the top remove the top cover of the proceedings

- Unscrew the screw from the bottom of the lower housing management

- Top two screws to allow the lower housing management

- Remove the bottom cover management

- Control lever to slide out after pressing the two plastic latches on the sides

The extension wires into the engine compartment:

The engine compartment can be reached in several ways, namely:

- Around the Bowden control opening bonnet

- Rubber grommet over the pedals

- Around the internal heating pipes

Some vehicles are already wire from the control unit can be located on the wiring harness connector, which is led from the engine bay into the interior of the vehicle. It's a good idea to verify the installation of a new campaign to save wire. However, the connection of the connector X2, where the driver is taken out from the control unit is quite complicated, because the connector is poorly accessible. In the case of wire drawing looks best stretch wire around the brake cable control for opening the bonnet or even better around the pipe internal heat, which is slightly permeable foam. The extension is appropriate to use a heavier wire, which at the end sands and then puncture the rubber (foam), the outer casing (pipe) passes into the engine compartment. In the engine compartment to put the best car driver ”goose neck“ and attach the wires to the other route via pull-down strap.

Connecting the BC (for cars with 12- pin connector):

The original connector of tidu is designed to be inserted directly into the onboard computer module. The 32-pin connector on the module board computer connect other wires needed, depending on what all will be installed. Accordingly, you can also order the appropriate harness. For a basic BC just plug the following section:

32-pin Connector on BC		Connect to:				    Basic connection

-----------------------------------------------------------------------------------------------------------------

Pin n. 1 (GND)			The control lever, brown wire

Pin n. 6 (RPM)			The instruments onboard, Pin n. 20, green wire

Pin n. 23 (Button RESet)		The control lever, white wire

Pin n. 24 (Button SET)		The control lever, red wire

Pin n. 26 (Fuel)			The instruments onboard, Pin n. 25, blue-black wire

Pin n. 27 (ECU)			According to table the engine control unit

For some control functions

-----------------------------------------------------------------------------------------------------------------

Pin n. 15 (switch brake)		Switch brake (12V when you press the brake pedal)

Pin n. 16 (switch brake)		Switch brake (12V after turning the key in the ignition)

Pin n. 31 (coolant indicator)		The instruments onboard, Pin n. 28, blue wire

The remaining control functions

-----------------------------------------------------------------------------------------------------------------

Pin n. 13				Oil Level

Pin n. 17				Check Control, Pin n. 1

Pin n. 18				Check Control, Pin n. 2

Pin n. 19				Brake pads

Pin n. 20				Coolant Level

Pin n. 22				Washer Level

Mechanically attached BC is just like the original MID or TID. Before tightening the screws and bolts PP vystředíme tighten. Cabling to prepare, covers insulating tape, or put in the "goose neck" and keep the margin for easy handling and insertion into the connector module BC after stretching and all connections. Wires leading to the devices should be plugged into some sleeves. Wire connection for devices with self-tapping perform connections or soldering and then " dragging" shrink sleeve.

Wires to the control lever can be connected directly, but in cases where the levers it can cause complications. Therefore, it is preferable to use a 3-pin connector. Original counterpart to the control lever, however, is looking bad.

The engine management system:

# Engine Type			Type ECU:		PIN to the ECU

X12XE / 1998			Motronic 1.5.5	PIN n. 16

X14XE / X16XEL		Multec			PIN n. 21 / X53

X16SZR			Multec			PIN n. 9

X18XE / X20XEV		Simtec			PIN n. 5

X17DTL / X20DTL		EDC 15M		PIN n. 29

Y17DTL / Y20DTH		EDC 15M		PIN n. 29

Y16XE / Z16XE		Multec			PIN n. 47 / X71

Z18XE				Simtec			PIN n. 20 / X57

Z22SE				Multec			PIN n. 8 / X77

Z12XE				Motronic 1.5.5	PIN n. 52 / X79

Y14XE / Z16SE		Multec			PIN n. 47 / X71

Y20DTL / Y20DTH		EDC (PSG5)		PIN n. 29 / X59

Y20DTH / Y22DTR		EDC (PSG16)		PIN n. 26 / X93

Z20LET			Motronic		PIN n. 52 / X79

Y17DT						PIN n. 47 / X73

Z16XEP			Multec			PIN n. 47 / X71

Z17DT	 (Z17DTL)					PIN č. 12 / X129

Z14XE				Multec			PIN n. 47 / X71

Wire color, if it is installed it is brown.

The listing pin engine control unit based on the documentation I have to install the MID, which was listed by production models 2002nd It is likely that they will pay for subsequent years of production, but it is necessary to check the mail. documentation for the model of the vehicle. Engine control unit X12XE since 1999 have reportedly preparing for the onboard computer. In this case the wire connected directly to the fuel injection valve, but you must use the isolating optocoupler. The install somewhere in the engine compartment.

If better alternatives will already be a pin on the control unit will be assembled and connected it to a conductor. In this case, terminate and connect the wire to the wire from pin-board computer module. The remaining wire that was originally connected to the insulated pin and leave it unconnected. Vehicles in the 2000/2001 production year around are usually fitted with a pin and the wire is terminated at connector X2. Vehicles made later, the driver usually do. It seems, began to investigate every meter cable.

If the pin connector on the control unit is not equipped at all, it should be added. A suitable option appears to be the pin of the 15-pin serial connector / female. Top of the middle line, because the pin is longer. The cable is to be soldered to pin. Then drag the pin shrink tubing. But we do not shrink tubing over the pin because the pin is a small band and drop through the tubing This crest would have prevented us from inserting the pin into the connector (see photo). Before inserting the pin hole should be cleansed of such small bit of silicon, silicon which pull out.

With proper installation procedure, the risk of damage to a minimum. Outlets control units are usually resistant to short circuits to the frame of the vehicle, but the damage could send them a direct connection board voltage, ie voltage battery. The driver of the PA module is connected internally to 12V 10K resistor, so that in case of bad connection leads to another, thanks to the ECU pin low power threaten damage to the outlet of the control unit.

Input circuit-board computer module for input from the control unit includes a ”flip-flop schmitt“ Hysteresis with a relatively large, so it is resistant to interference. However, it is possible to use shielded cable, which could be further eliminate possible interference. When using shielded cable, shielded only involved in the connector module on-board computer. The controllers do not attach the shield.

Connecting climate control:

The climate control will use a miniature relay is closing his practice simulates switch button on the control panel. Colors of wires and turn on the connector can vary by type. Before connecting the wires is necessary to check their accuracy. Top with a voltmeter / ohmmeter. The drivers for the climate control has to be 12V when you press the button climate control. After removing the connector and measuring the ohmmeter to the connector must be between the pins by pressing the right buttons for climate control fault.

Air Behr: 	Pin 11 red wire 4 mm

Pin 4 purple wire 0.5 mm

Air Delphi: 	Connector X65 / 5 black wire 4 mm

Connector X66 / 2 black / green wire 0.5 mm

If the outlet for the climate control is available (the proceedings are dealt with by other means), it is necessary to remove control panel with buttons and connect directly to the relay switch, which switches the air conditioning.

After connecting all cables to connect to the onboard computer module connector from the original TID, plug the wire from the control unit from the onboard instruments, the operating lever and can proceed to completion. On-board computer module assembly increasing the depth of the screen, so you must cut a hole in a paper cover for the display, which would otherwise be the module on-board computer and the screen pushed and could not be assembled center console.

Installing the Opel Frontera

Installation in the vehicle Frontera is very simple thanks to the connector X26 , located in the center console on the radio . Further tightening of wires is necessary only for certain control functions . Optimal would be to find a mating connector X26 for some volume. But this is quite difficult, so do nothing else but the wire connector X26 interrupt and connect directly.

32-pin Connector on BC		Connect to:				    Basic connection

-----------------------------------------------------------------------------------------------------------------

Pin n. 6 (RPM)			Connector X26/1 (black-red wire)

Pin n. 23 (Button RESet)		Connector X26/7 (vhite-black wire)

Pin n. 24 (Button SET)		Connector X26/8 (white-red wire)

Pin n. 26 (Fuel)			Connector X26/9 (yellow-red wire)

Pin n. 27 (ECU)			Connector X26/10 (green wire)

For some control functions

-----------------------------------------------------------------------------------------------------------------

Pin n. 15 (switch brake)		Switch brake (12V when you press the brake pedal)

Pin n. 16 (switch brake)		Connector X26/3

The remaining control functions

-----------------------------------------------------------------------------------------------------------------

Pin n. 13 (Oil level)			Connector X26/2

Pin n. 17				Check Control, Pin n. 1

Pin n. 18				Check Control, Pin n. 2

Pin n. 19 (Brake pad)			Connector X26/4 (green-black wire)

Pin n. 20 (Coolant level)		Connector X26/5 (green-red wire)

Pin n. 22 (Washer level)		Connector X26/6 (light green wire)

Climate control is located at the air-conditioning switches S47

Pin n. 2 Brown wire 0.5 mm

Pin n. 1 Green-black wire 0.5 mm

Any other questions like the answer at veramon.astra@seznam.cz