Installation of the On-Board computer Astra G

The module is installed on the imaging display (TID), which is located in the upper center console, which must therefore be removed. It is necessary to remove the module on-board devices, where the power cable assembly connects the measurement of engine speed and the condition of the fuel tank, then stretch and attach wire to the engine control unit and replaced the right control stick for steering. Before the installation is recommended that you disconnect the car battery.

Removing the center console:

- Remove the radio and storage space (the drawer under the car radio)

- Remove ashtray and remove the cover of a car cigarette lighter

- Allow the screws under the cover of a car ashtray and lighter

- Remove the metal shaft radio (on the back is screw)

- Cut to the car stereo connectors, including the antenna connector

- Allow the screws on the sides, where they found the shaft radio

- Disconnect the control air-conditioning (2 clips at the top and bottom 2 clips)

- Pull switch to remove the directional warning lights

Now we no longer cover the entire center console holds the upper metal clip, making it stronger by pulling out and then pull out.

- Push out the plug warning light

- Disconnect connector from the display TID

Removing the panel board instruments:

- Take off two covers at the bottom of the flight instrument

- Allow four screws (2 below and 2 above)

- Push out the plastic cover on-board instruments

- Allow the screws that hold the board device (2 below)

- Disconnect connector (top left of the lever is plastic)

- Remove the panel board instruments

- Remove the connector cable assembly from the cradle

To remove the connector to the cable assembly from the holder, it is necessary to squeeze plastic holder at the bottom left downwards. Connector beam can then be easily pulled to the left out

Removing control

- Pull the two plastic screw covers on the steering wheel (it is necessary to turn the steering wheel)

- Unscrew the screws underneath these covers

-  Towards the top remove the top cover of the proceedings

- Unscrew the screw from the bottom of the lower housing management

- Top two screws to allow the lower housing management

- Remove the bottom cover management

- Control lever to slide out after pressing the two plastic latches on the sides

The extension wires into the engine compartment:

The engine compartment can be reached in several ways, namely:

- Around the Bowden control opening bonnet

- Rubber grommet over the pedals

- Around the internal heating pipes

Some vehicles are already wire from the control unit can be located on the wiring harness connector, which is led from the engine bay into the interior of the vehicle. It's a good idea to verify the installation of a new campaign to save wire. However, the connection of the connector X2, where the driver is taken out from the control unit is quite complicated, because the connector is poorly accessible. In the case of wire drawing looks best stretch wire around the brake cable control for opening the bonnet or even better around the pipe internal heat, which is slightly permeable foam. The extension is appropriate to use a heavier wire, which at the end sands and then puncture the rubber (foam), the outer casing (pipe) passes into the engine compartment. In the engine compartment to put the best car driver ”goose neck“ and attach the wires to the other route via pull-down strap.

Wiring board computer

Module on-board computer with black insert, 12-pin connector to the display (tidu). Connector on the module is smaller than the hole on the display, so we make sure that the connector was in the middle and not significantly shifted to one side. Connector plug is quite tough, so we push directly on the connector pins and not the county board module! The orientation of the display module is shown in the photo, which is located on the website. The module is an inlet that serves to fix the module to display with the screws. When mounting screw subsoil plastic washer between the board and display module, we attached a plastic column. 

 	Prepare the external connector to the onboard computer module, which wires lead to the flight instrument panel, engine compartment and the control lever of control. Coat wires with insulation tape or put the car into the ”goose neck“. We will prepare you plug into the assembly area and keep the wire length cushion for easy handling with the connector. Wires stretch to places where they are attached, and fasten them on route to other wires using cable tapes. Conductors for scanning speed and the fuel leading to the onboard motherboard devices should be plugged into any ”tubes“.

Onboard instruments :

Speedometer – No. 20 on the connector onboard instruments (green wire)

Measurement of fuel – No. 25 on the connector onboard instruments (blue/black wire)

Control Stick:

Tl. RESET - A pin on the control lever (white)

Tl. SET -  D pin on the control lever (red)

GND (frame) – C pin on the control lever (brown)

Wires to the control lever can be connected directly, but in cases where the levers it can cause complications. Therefore, it is preferable to use a 3-pin connector. Original counterpart to the control lever, however, is looking bad.

The engine management system:

# Engine Type::		     	Type ECU:		PIN to the ECU

X12XE / 1998			Motronic 1.5.5	PIN č. 16

X14XE, X16XEL		Multec			PIN č. 21 / X53

X16SZR			Multec			PIN č. 9

X18XE, X20XEV		Simtec			PIN č. 5

X17DTL, X20DTL		EDC 15M		PIN č. 29

Y17DTL, Y20DTH		EDC 15M		PIN č. 29

Y16XE, Z16XE		Multec			PIN č. 47 / X71

Z18XE				Simtec			PIN č. 20 / X57

Z22SE				Multec			PIN č. 8 / X77

Z12XE				Motronic 1.5.5	PIN č. 52 / X79

Y14XE, Z16SE		Multec			PIN č. 47 / X71

Y20DTL, Y20DTH		EDC (PSG5)		PIN č. 29 / X59

Y20DTH, Y22DTR		EDC (PSG16)		PIN č. 26 / X93

Z20LET			Motronic		PIN č. 52 / X79

Y17DT						PIN č. 47 / X73

Z16XEP			Multec			PIN č. 47 / X71

Z17DT, Z17DTL					PIN č. 12 / X129

Z14XE				Multec			PIN č. 47 / X71

Wire color, if it is installed it is brown/black.

The listing pin engine control unit based on the documentation I have to install the MID, which was listed by production models 2002nd It is likely that they will pay for subsequent years of production, but it is necessary to check the mail. documentation for the model of the vehicle. Engine control unit X12XE since 1999 have reportedly preparing for the onboard computer. In this case the wire connected directly to the fuel injection valve, but you must use the isolating optocoupler. The install somewhere in the engine compartment.

If better alternatives will already be a pin on the control unit will be assembled and connected it to a conductor. In this case, terminate and connect the wire to the wire from pin-board computer module. The remaining wire that was originally connected to the insulated pin and leave it unconnected. Vehicles in the 2000/2001 production year around are usually fitted with a pin and the wire is terminated at connector X2. Vehicles made later, the driver usually do. It seems, began to investigate every meter cable.

If the pin connector on the control unit is not equipped at all, it should be added. A suitable option appears to be the pin of the 15-pin serial connector / female. Top of the middle line, because the pin is longer. The cable is to be soldered to pin. Then drag the pin shrink tubing. But we do not shrink tubing over the pin because the pin is a small band and drop through the tubing This crest would have prevented us from inserting the pin into the connector (see photo). Before inserting the pin hole should be cleansed of such small bit of silicon, silicon which pull out.

With proper installation procedure, the risk of damage to a minimum. Outlets control units are usually resistant to short circuits to the frame of the vehicle, but the damage could send them a direct connection board voltage, ie voltage battery. The driver of the PA module is connected internally to 12V 10K resistor, so that in case of bad connection leads to another, thanks to the ECU pin low power threaten damage to the outlet of the control unit.

Input circuit-board computer module for input from the control unit includes a ”flip-flop schmitt“ Hysteresis with a relatively large, so it is resistant to interference. However, it is possible to use shielded cable, which could be further eliminate possible interference. When using shielded cable, shielded only involved in the connector module on-board computer. The controllers do not attach the shield.

Connecting climate control:

The climate control will use a miniature relay is closing his practice simulates switch button on the control panel. Colors of wires and turn on the connector can vary by type. Before connecting the wires is necessary to check their accuracy. Top with a voltmeter / ohmmeter. The drivers for the climate control has to be 12V when you press the button climate control. After removing the connector and measuring the ohmmeter to the connector must be between the pins by pressing the right buttons for climate control fault.

Air Behr: 	Pin 11 red wire 4 mm

Pin 4 purple wire 0.5 mm

Air Delphi: 	Connector X65 / 5 black wire 4 mm

Connector X66 / 2 black / green wire 0.5 mm

If the outlet for the climate control is available (the proceedings are dealt with by other means), it is necessary to remove control panel with buttons and connect directly to the relay switch, which switches the air conditioning.

After connecting all cables to connect to the onboard computer module connector from the original TID, plug the wire from the control unit from the onboard instruments, the operating lever and can proceed to completion. On-board computer module assembly increasing the depth of the screen, so you must cut a hole in a paper cover for the display, which would otherwise be the module on-board computer and the screen pushed and could not be assembled center console.

Installing the Opel Zafira

Due to similarities with the Opel Astra G installations electrically identical. Same is the wiring to the speed and the fuel to on-board devices, as well as connecting wires to the control unit and engine control lever. Different involvement of the module on-board computer, which, due to lack of space for TIDE must be placed behind a metal construction and the TIDE is connected via ribbon cable. The link is clearly shown in the picture. When installation is necessary to pay attention to the numeric designation order to prevent reverse insertion of the connectors. Numbering module pin-board computer is also shown in the image numbering TID connectors directly to the stamping of connectors.

Installation of the Opel Vectra B Omega B

Opel Vectra B Opel Omega B uses the display connector with the same TID as the Opel Zafira, so this module can be connected without complications well these vehicles. Different is connected to the onboard motherboard devices, where the outputs of tachometer and fuel level located on the other pins.

When installing, make sure to correctly insert the connectors! Instead, all carefully checked to avoid damage to the module. You can easily orient the picture, which is depicted sequence tags pin connectors.

Onboard instruments Vectra B :

Speedometer – No. 3 on the connector onboard instruments (green wire)

Measurement of fuel – No. 1 on the connector onboard instruments (blue/black wire)

Onboard instruments Omega B :

Speedometer – No. 22 on the connector onboard instruments (green wire)

Measurement of fuel – No. 20 on the connector onboard instruments (blue/black wire)

The engine controller Opel Vectra B

# Engine Type:			ECU:			PIN to the ECU

Z18XE				Simtec			X67/20

X18XE			Simtec			pin č. 21

Y20DTH, Y22DTR		Diesel			X80/29

Z22SE				Multec			X69/8

Z16XE				Multec			X53/47

X16XEL			Multec			X45/21

X20XEV			Simtec			pin č. 29

Wire color, if it is installed it is brown.

The engine controller Opel Omega B

# Engine Type:			ECU:			PIN to the ECU

Z22XE				Simtec			X84/20

Y26(32)SE			Motronic		X86/3

Y22DTH			Diesel			X80/29

Y25DT			Diesel CR		X73/16

Wire color, if it is installed it is brown.

Installing Opel Frontera

By Vehicle Vauxhall Frontera PA module can be used as well as in the Astra G. The installation is somewhat easier with the X26 connector, located on the right side of the center console around the radio.

X26 Connector:

Speedometer:  		X26 / 1 (black/red wire)

Measurement of fuel:  	X26 / 9 (yellow/red wire)

Engine control unit: 		X26/10 (green wire)

Button RESET:		X26 / 7 (black/white wire)

Button SET 			X26 / 8 (white/red wire)

Climate control is a switch S47 air conditioning :

Pin No. 2 Brown wire 0.5 mm

Pin No. 1 Grenn/Black wire 0.5 mm

Installing Opel Corsa B

The car Opel Corsa B can use the same module as the Astra F. Involvement of flight instrument panel is as follows:

Onboard instruments Corsa B :

Speedometer – No. 9 on the connector onboard instruments (green wire)

Measurement of fuel – No. 18 on the connector onboard instruments (blue/black wire)

Opel Corsa B is produced in the version with the onboard computer, so to capture information on fuel consumption is necessary to use some of the injection valve and the entrance to the module on-board computer to provide a simple optocoupler. Photo optocoupler scheme is shown on the website.

Technical parameters:

Dimensions: 				57 x 35 mm x 15 mm

Power: 				12V (14V)

Consumption (standby): 		Approx 5.5 mA

Max. current for external relay: 	100mA

Any other questions like the answer at veramon.astra@seznam.cz